AirLive RTSP Specification (V6)

Content

1.    DOCUMENT HISTORY

2.    1.1 Product and firmware versions

3.    3.1 RTSP DESCRIBE

4.    3.2 RTSP OPTIONS

5.    3.3 RTSP SETUP

6.    3.4 RTSP PLAY

7.    3.5 RTSP TEARDOWN

8.    3.6 RTSP SET_PARAMETER

9.    3.6 RTSP over HTTP

DOCUMENT HISTORY

| Version | Date | Supported Firmware | Release Notes |
| --- | --- | --- | --- |
| 1.00 | 2007-May-14 | V3.0.0.0 | Initial version |
| 1.01 | 2007-Dec-28 | V3.0.2.1691 | Support Mega-pixel streaming |
| 1.02 | 2008-Mar-04 | V3.0.2.1829 | Corrected errors in examples |
| 1.03 | 2008-May-20 | V5.0.0.0 | F/W version 5.0 which supports H.264 streaming |
| 1.04 | 2008-Sep-26 | V5.0.0.2565 | New RTSP URL format |
| 1.05 | 2010-Jan-04 | V5.0.2.3801 | ROI URL |
| 1.06 | 2010-Sep-07 | V5.0.2.3801 | Remove Pause command |
| 1.07 | 2010-Mar-17 | V5.0.2.5746 | Add SET_PARAMETER command |
| 2.0 | 2011-Apr-11 | V6.X.2.5917 | Add streamprofile paramter |

1 OVERVIEW

This document specifies the external RTSP-based application programming interface of the AirLive cameras with firmware version

3.00 and above.

1.1 Product and firmware versions ¶

The support for the RTSP API is product and firmware dependent. Please refer to the Release Notes for the actual product for compliance information.

2 REFERENCES

       RTSP protocol

o Real Time Streaming Protocol - RFC 2326

       SDP protocol

o Session Description Protocol - RFC 2327

       HTTP protocol

o Hypertext Transfer Protocol -- HTTP/1.0

       External application programming interfaces (Client side)

o AirLive Video HTTP API

o AirLive Video RTSP API

o AirLive Video Parameter specification

3 RTSP API

RTSP URL format has following type:

rtsp://<servername>/media/media.amp

[?<parameter>=<value>[&<parameter>=<value>...]]

The following parameters are supported for H.264, MPEG-4 Part 2 and MJPEG streams.

| Parameter | Valid values | Description |
| --- | --- | --- |
| streamprofile | Profile name, must the same as Image.I0.Appearance#.Name | Used in V6 firmware version. |

The DESCRIBE, SETUP, OPTIONS, PLAY, SET_PARAMETER and TEARDOWN methods are supported. The RTSP protocol is described in RFC 2326.

Request syntax:

COMMAND  URI RTSP/1.0<CRLF>

Headerfield1: val1<CRLF>

Headerfield2: val2<CRLF>

...

<CRLF>

Response syntax:

RTSP/1.0 ResultCode  ResultString<CRLF>

Headerfield3: val3<CRLF>

Headerfield4: val4<CRLF>

...

<CRLF>

The following header fields are accepted by all commands. Other header fields are silently ignored (unless stated otherwise in the

sections below).

| Field | Description |
| --- | --- |
| Authorization | Authorization information from the client. |
| CSeq | Request sequence number. |
| Session | Session identifier (returned by server in SETUP response). |
| Content-Length | Length of content. |

The following header fields can be generated for all responses by the RTSP server:

| Field | Description |
| --- | --- |
| CSec | Response sequence number (matches the sequence number of the request). |
| Session | Session identifier. |
| WWW-Authenticate | Authentication from client requested. |

3.1 RTSP DESCRIBE

       The DESCRIBE command returns the SDP (RFC 2327) description for the URI.

       The DESCRIBE command accepts the following additional header fields:

| Field | Description |
| --- | --- |
| Accept | List of content types that client supports (application/sdp is the only supported type). |

The DESCRIBE command generates the following additional header fields:

| Field | Description |
| --- | --- |
| Content-Type | Type of content (application/sdp). |
| Content-Length | Length of SDP description. |
| Content-Base | If relative URLs are used in the SDP description, then this is the base URL. |

Example: ¶

DESCRIBE  /mpeg4/1/media.amp  RTSP/1.0

CSec:  1

Accept: application/sdp

Authorization: Basic   cm9vdDpwYXNz

Response example:

RTSP/1.0 200  OK

CSec:  1

Content-Base: rtsp://x.y.z.w:554/mpeg4/quad/media.amp

Content-Type: application/sdp

Content-Length: 680

v=0

o=-  1109162014219182  1109162014219192  IN IP4  x.y.z.w

s=Media Presentation

e=NONE

c=IN IP4  0.0.0.0

b=AS:8000

t=0  0

a=control:*

a=range:npt=now-

a=mpeg4-iod:

"data:application/mpeg4-iod;base64,AoDUAE8BAf/1AQOAbwABQFBkY XRhOmFwcGxpY2

F0aW9uL21wZWc0LW9kLWF1O2Jhc2U2NCxBUjBCR3dVZkF4Y0F5U1FBWlFRTk

lCRUVrK0FBZWhJQUFIb1NBQVlC

QkE9PQQNAQUABAAAAAAAAAAAAAYJAQAAAAAAAAAAAzoAAkA2ZGF0YTphcHBs

aWNhdGlvbi9tcGVnNC1iaWZzLW

F1O2Jhc2U2NCx3QkFTWVFTSVVFVUZQd0E9BBICDQAAAgAAAAAAAAAABQMAAE

AGCQEAAAAAAAAAAA=="

m=video 0  RTP/AVP  96

b=AS:8000

a=control:trackID=1

a=rtpmap:96 MP4V-ES/90000

a=fmtp:96  profile-level-id=245;

config=000001B0F5000001B509000001000000012008D495880325

0B042414440F;

a=mpeg4-esid:201

3.2 RTSP OPTIONS

The OPTIONS command returns a list of supported RTSP commands.

Example:

OPTIONS  *  RTSP/1.0

CSec:  2

Response example:

RTSP/1.0 200  OK

CSec:  2

Public: DESCRIBE,  PLAY,  SETUP, TEARDOWN

3.3 RTSP SETUP

The SETUP command configures the delivery method for the data. The SETUP command requires and generates the following additional header field:

| Field | Description |
| --- | --- |
| Transport | Specifies how the data stream is transported. |
|  | Supported variants: |
|  | RTP/AVP;unicast;client_port=port1-port2 |
|  | RTP/AVP;multicast;client_port=port1-port2 |
|  | RTP/AVP/TCP;unicast |

The response returns a session identifier that should be used with stream control commands to the server (PLAY, TEARDOWN). If the Session header includes a timeout parameter, then the session needs to be kept alive. This can be done by sending RTSP requests to the server containing the session identifier (e.g. OPTIONS) within the specified timeout time or through the use of RTCP. The RTSP server does not support reconfiguring of the transport parameters.

Example:

SETUP  /mpeg4/1/media.amp/trackID=1  RTSP/1.0

CSeq: 3

Transport:  RTP/AVP;unicast;client_port=20000-20001

Response example:

RTSP/1.0 200  OK

CSeq: 3

Session:  1442774578;timeout=60

Transport:

RTP/AVP;unicast;mode=play;server_port=49000-49001;client_por t=20000-20001

3.4 RTSP PLAY

The PLAY command starts the data delivery to the client. The PLAY command generates the following additional header fields:

| Field | Description |
| --- | --- |
| Range | Specifies the range of time being played. Since only live streams are used, the specified time will always begin now and have no stop time. |
| RTP-Info | Information about the RTP stream. More specifically, it includes the next RTP sequence number that will be used. |

Example:

PLAY  /mpeg4/1/media.amp  RTSP/1.0

CSeq: 4

Session: 1442774578

Response example:

RTSP/1.0 200  OK

CSeq: 4

Session: 1442774578

Range:  npt=now-

RTP-Info: url=trackID=1;seq=2735

3.5 RTSP TEARDOWN

The TEARDOWN command terminates the data delivery from the server.

Example:

TEARDOWN  /mpeg4/1/media.amp  RTSP/1.0

CSeq: 6

Session: 1442774578

Response example:

RTSP/1.0 200  OK

CSeq: 6

Session: 1442774578

3.6 RTSP SET_PARAMETER

The SET_PARAMETER command is used to request an I-frame, for example when starting a recording. The command sets the value of the Renew-Stream parameter to yes. Renew-Stream must be sent in the body or in the header.

Example (in the body):

SET_PARAMETER  rtsp://myserver/axis-media/media.amp  RTSP/1.0

CSeq: 7

Session: 12345678

Authorization: Basic   cm9vdDpwYXNz

Content-Type: text/parameters

Content-Length: 19

Renew-Stream:  yes

Example (in the header):

SET_PARAMETER  rtsp://myserver/axis-media/media.amp  RTSP/1.0

CSeq: 7

Session: 12345678

Authorization: Basic   cm9vdDpwYXNz

Require: com.axis.parameters-in-header

RenewStream:  yes

Response example:

RTSP/1.0 200  OK

CSeq: 7

Session: 12345678

Date: Wed, 16  Jul 2008  13:01:25 GMT

3.6 RTSP over HTTP

It is possible to tunnel RTSP through HTTP to get through firewalls etc. This is achieved by setting up two sessions, one GET (for command replies and stream data) and one POST (for commands). RTSP commands sent on the POST connection are base64 encoded, but replies on the GET connection are plain text. To bind the two sessions together the server needs a unique ID (conveyed in the x-sessioncookie header). The GET and POST request are accepted both on the HTTP port (default 80) and the RTSP server port (default 554). The RTSP/HTTP URL is: http//<server name>/mpeg4/1/media.amp. Supported methods are GET and POST.

GET example:

GET  /mpeg4/1/media.amp HTTP/1.0

x-sessioncookie: 944345808

Response example:

HTTP/1.0 200  OK

Content-Type: application/x-rtsp-tunnelled

Note that the returned Content-Length is an arbitrary number and does not indicate the size of any data.

POST example:

POST  /mpeg4/1/media.amp HTTP/1.0

x-sessioncookie: 944345808

Content-Length: 32767

Content-Type: application/x-rtsp-tunnelled

Note that the x-sessioncookie present in the POST content is the same as the x-sessioncookie in the corresponding GET request. Also note that the POST request must be sent on a separate connection to the server (i.e. it cannot be sent on the same connection as the GET request). No response is sent for the POST request.

Command example (base 64 encoded PLAY on the POST connection):

UExBWSAvbXBlZzQvMS9tZWRpYS5hbXAgUlRTUC8xLjANCkNTZXE6IDQNClNl

c3Npb246IDEzNjk5MTc3MTINCg0K

Which corresponds to this command:

PLAY  /mpeg4/1/media.amp  RTSP/1.0

CSeq: 4

Session: 1369917712

Command response (plain-text on the GET connection):

RTSP/1.0 200  OK

CSeq: 4

Session: 1369917712