IP Camera Open Parameter Specification

DOCUMENT HISTORY

| Version | Date | Supported Firmware | Release Notes |
| --- | --- | --- | --- |
| 1.00 | 2007-May-10 | V3.0.0.0 | Initial version. |
| 1.01 | 2007-Sep-27 |  | Add focus and autofocus properties. |
| 1.02 | 2007-Nov-02 |  | 1. Modify duration of "Event HW Actions" |
|  |  |  | 2. Modify Event "Voice Alert Actions" |
|  |  |  | 3. Modify Event Guard Tour Actions |
| 1.03 | 2007-Dec-12 | V3.0.2.1613 | 1. Implement Image.I0.Stream |
|  |  |  | 2. Implement Image.I0.Appearance.Rotation |
|  |  |  | 3. Remove ImageSource.I0.Sensor.Mirror |
|  |  |  | 4. Remove ImageSource.I0.Sensor.Flip |
|  |  |  | 5. Implement Layout.DefaultVideoFormat |
|  |  |  | 6. Modify Image.I0.Stream.FPS |
| 1.04 | 2007-Dec-18 | V3.0.2.1628 | 1. Add Audio.A0.HTTPMessageType |
|  |  |  | 2. AudioSource.A0.AudioEncoding=g721 |
| 1.05 | 2007-Dec-28 | V3.0.2.1691 | Revise Document |
| 1.06 | 2008-Jan-04 | V3.0.2.1691 | Revise Document |
| 1.07 | 2008-Apr-02 | V3.0.2.1918 | Add tcp and http event action |
| 1.07 | 2008-Apr-02 | V3.0.2.1918 | Add tcp and http event action |
| 2.00 | 2008-Mar-27 | V5.0.0.0 | H.264 Initial version. Mark as **h264. |
| 2.01 | 2008-Sep-26 | V5.0.0.2565 | H.264 Modify Image.I0.Appearance parameter |
|  |  |  | H.264 Modify Motion.M# parameter |
|  |  |  | H.264 Add IOPort parameter |
| 2.02 | 2009-Mar-05 | V5.0.0.3006 | Add/Modify Image.I0.Appearance parameter |

| 2.03 | 2009-May-27 | V5.0.0.3260 | Add audio mute while P/T |
| --- | --- | --- | --- |
| 3.00 | 2009-Dec-10 | V3.0.5.3714 V5.0.2.3714 V5.a.2.3714 | Add SIP parameter. |
| 3.01 | 2009-Dec-23 | V3.0.5.3714 V5.0.2.3714 V5.a.2.3714 | Improve event action parameters. |
| 3.02 | 2010-Jan-07 |  | Increase Properties.DynamicParam parameters (2.11.19) |
|  |  |  | Increase ROI parameters : 2.2.3 Image.I0.Appearance |

1 OVERVIEW

This document specifies the parameters for all IP cameras and video servers.

1.1 Product and firmware versions

The parameters specified in this document could be applied to all products with firmware V3.00.

1.2 Security levels

The security levels column in each parameter table shows the required security level to operate parameters. The numbers represent security levels on the form create/delete for groups and get/set for parameters. To be able to perform an action on a parameter the user needs to have a security level equal to or higher than the corresponding security level of the parameter.

| Security level | Description |
| --- | --- |
| 0 | Unprotected, but it is not possible to access the camera setting without at least view rights. |
| 1 | View access right needed to access the parameter. |
| 6 | Administrator access right needed to access the parameter. |

Internal parameters that can only be changed by firmware

7

applications or by root.

1.3 Valid values

The following valid values are used in this document:

| Valid values | Description |
| --- | --- |
| An integer | Any number between -2147483647 (-231-1) and 2147483647 (231-1). |
| An unsigned integer | Any number between 0 and 4294967295 (232-1). |
| <m> | Any number starting from number m. |
| <m> ... <n> | Any number between number m and number n. |
| A string | Any string (valid characters: ISO 8859-1). |
| A domain name | A string limited to contain a domain name. |
| A host name | A string limited to contain a host name. |
| An IP address | A string limited to contain an IP address of the format xxx.xxx.xxx.xxx, where xxx is a number between 0 to 255. Example: 192.168.0.90 |
| A MAC Address | A string limited to contain a MAC address of the format xx:xx:xx:xx:xx:xx, where xx is a hexadecimal value. Example: 00:40:8c:cd:00:00 |
| An e-mail address | A string limited to contain an e-mail address. |
| A URL/URI | A sting limited to contain a URL/URI. |
| A path | A string limited to contain a path. |
| A time | A string limited to contain a time of the format hh:mm:ss. Example: 23:01:14 |
| A date | A string limited to contain a date of the format yyyy-mm-dd. Example: 2004-02-16 |
| <value 1><value 2><value 3>... | Enumeration, only the given values are valid. Example:yes no |
| <m><value> ... <n><value> <value><m> ... <value><n> | Any number between m and n together with value. Example: 1Mbit ... 100Mbit |
| Read only | Only the default value is valid as value. |

| Auto generated | Automatically generated value, should not be changed manually. |
| --- | --- |
| Hardware dependent | The hardware decides the default value/the valid values. |
| Everything inside brackets | Description. |

2 PARAMETER GROUPS

2.1 General

2.1.1 Brand

| Description: Contains information about the brand, name and type of the product. [Brand] Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Brand | AirLive | A string (Auto generated) | 0/7 | The brand of the product. |
| ProdFullName | * | A string (Auto generated) | 0/7 | The full name of the product. |
| ProdNbr | * | A string (Auto generated) | 0/7 | The product number. |
| ProdShortName | * | A string (Auto generated) | 0/7 | The short name of the product. |
| ProdType | * | Video Server, Network Camera, Network Video Recorder (Auto generated) | 0/7 | The product type. |
| WebURL | http://www.airlive.com | A string (Auto generated) | 0/7 | The URL to visit for |

|  |  |  |  | support and information about the product. |
| --- | --- | --- | --- | --- |

2.1.2 Network

Description: Network interface settings. The parameters in this group (as opposed to the subgroups of this group) are static network settings. If the Network.BootProto parameter is "dhcp" these parameters may not be in use so always use the read-only parameters in the subgroups to retrieve actual network settings in use by the operating system.

[Network]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| BootProto | dhcp | dhcp, none | 6/6 | Enable/disable dynamic IP address assignment to the device. |
| Broadcast | 192.168.0.255 | An IP address | 6/6 | Broadcast address. Used to disseminate information to several recipients simultaneously. |
| DefaultRouter | 192.168.0.254 | An IP address | 6/6 | Default router/gateway used for connecting devices attached to different networks and network segments. |
| DNSServer1 | 0.0.0.0 | An IP address | 6/6 | Primary Domain Name System server. |
| DNSServer2 | 0.0.0.0 | An IP address | 6/6 | Secondary Domain Name System server. |
| DomainName |  | A domain name | 6/6 | The name of the domain to which the device belongs. |

| InterfaceSelectMode | auto | auto, wired | 6/6 | Interface select mode. Defines how the device chooses which network interface to use. In auto mode the wireless interface will be used when a wired network connection cannot be detected. In wired mode only the wired interface will be used, regardless of its' status. |
| --- | --- | --- | --- | --- |
| IPAddress | 192.168.0.90 | An IP address | 6/6 | IP Address. The physical address of the device on the network. |
| SubnetMask | 255.255.255.0 | An IP address | 6/6 | Subnet mask. Divides the network. |

Description: Network dynamic IP service. AirLive service to help to translate device

IP into number that could found in our website.

2.1.3 Network.Bonjour

Description: Enable/disable Bonjour and set the name to be displayed in

Bonjour-clients.

[Network.Bonjour]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Enabled | yes | yes,no | 6/6 | Enable/disable Bonjour. |
| FriendlyName | <product name> - <MAC> | A string | 6/6 | The name of the Bonjour device. |

2.1.4 Network.DNSUpdate

Description: Dynamic Updates in the Domain Name System (RFC 2136).

[Network.DNSUpdate]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| DNSName |  | absolute domain name | 6/6 | The name entered here will be associated with the product's IP address in the DNS server. |
| Enabled | no | yes, no | 6/6 | Enable/disable dynamic DNS service. |
| TTL | 30 | 0 ... <232-1> | 6/6 | This value determines how long (in seconds) the reply from the DNS server should be remembered when checking that the domain name for the registered IP address is still valid. This will reduce the amount of network traffic necessary for the unit's operation. |
| DNSSERVER | www.dyndns.org | <absolute domain name> | 6/6 | DDNS server |
| Username |  | a string username | 6/6 | username needed to login the DDNS server |
| Password |  | a string password | 6/6 | password needed to login the DDNS server |

2.1.5 Network.eth0

Description: Network settings of the first ethernet interface. Use these parameters to retrieve the network settings actually in use by the operating system.

[Network.eth0]

| Parameter name | Default value | Valid values | Security level | Description |
| --- | --- | --- | --- | --- |

|  |  |  | (get/set) |  |
| --- | --- | --- | --- | --- |
| Broadcast |  | An IP address (Auto generated) | 6/6 | Broadcast address. Used to disseminate information to several recipients simultaneously. This parameter is read only. |
| IPAddress |  | An IP address (Auto generated) | 6/6 | IP Address. The physical address of the device on the network. This parameter is read only. |
| MACAddress | 00:40:8c:xx:xx:xx* | A MAC address (Auto generated) | 6/6 | MAC address. The unique identity of the device. This parameter is read only. |
| SubnetMask |  | An IP address (Auto generated) | 6/6 | Subnet mask. Divides the network. This parameter is read only. |

2.1.6 Network.Filter

Description: IP address filtering. These parameters are used to only accept connections from certain IP addresses or networks.

[Network.Filter]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Enabled | no | yes,no | 6/6 | Enable/disable IP address filtering. |

2.1.7 Network.Filter.Input

Description: IP addresses filtering for incoming packets. [Network.Filter.Input]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| AcceptAddresses |  | A string (a space separated list of IP addresses and network addresses in the CIDR notation (IP address/netmask bits)) | 6/6 | Addresses allowed to pass through the filter. Example: 192.168.0/24 will add all the addresses in the range 192.168.1.1 to 192.168.1.254. Note: If accessing the device via a proxy server, the proxy server's IP address must be added to the list of allowed addresses. |
| Policy | allow | allow,deny | 6/6 | Allow or deny the addresses access to the device. |

2.1.8 Network.Interface.I#

Description: Configuration and status information for one network interface on an device.

[Network.Interface.I0]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Type | 802.3 | 802,3 | 6/6 | The type of the network interface as expressed by the IEEE 802 protocol. This value is read-only. |

[Network.Interface.I1]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Type | 802.11 | 802.11 | 6/6 | The type of the network interface as expressed by the IEEE 802 protocol. This value is read-only. |

2.1.9 Network.Interface.I#.Active

Description: Settings of this network interface. Use these parameters to retrieve the network settings actually in use by the operating system.

[Network.Interface.I#.Active]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| MACAddress | xx:xx:xx:xx:xx:xx | A MAC address | 6/6 | The MAC address of the device. |
| Active |  | yes,no | 6/6 | The in use status of the network interface. yes = the network is currently in use. |
| IPAddress |  | An IP address | 6/6 | The network address of this network interface on the device. SubnetMask An IP address 6/7 The subnet mask. |
| Broadcast |  | An IP address | 6/6 | The broadcast address. Used to disseminate information to several recipients simultaneously. |

2.1.10 Network.Interface.I#.Link

Description: The low-level configuration of a network interface. [Network.Interface.I#.Link]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| BootProto | dhcp | dhcp,none | 6/6 | Enable/disable dynamic IP assignment to this network interface on the device. Media auto auto, 10baseT-HD, 10baseT-FD, 100baseTX-HD, 100baseTX-FD 6/6 Media type on the network interface. Only available on Ethernet interfaces, i.e. if Network.Interface.I#.Type=802.3. |

2.1.11 Network.Interface.I#.Manual

| Description: The manual configuration of a network interface. [Network.Interface.I#.Manual]* Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| IPAddress | 192.168.0.100 | An IP address | 6/6 | The network address of this network interface on the device. |
| SubnetMask | 255.255.255.0 | An IP address | 6/6 | Subnet mask for the network interface. Divides the network. |
| Broadcast | 192.168.0.255 | An IP address | 6/6 | Broadcast address. Used to disseminate information to several recipients simultaneously. |
| DefaultRouter | 192.168.0.254 | An IP address | 6/6 | Default router/gateway used for connecting devices attached to different networks and networks segment. |

2.1.12 Network.Interface.I#.ZeroConf

Description: Enable/disable automatic configuration of link local IP address. The negotiated network settings are located in this group, and are used in parallel with the settings of the Network.Interface.I#.Active group.

I.e. both addresses can be used simultaneously.

[Network.Interface.I#.ZeroConf]*

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Enabled | yes | yes,no | 6/6 | Enable/disable zero configuration. |
| IPAddress |  | An IP address (Auto generated) | 6/6 | These values shall not be configured. They are retrieved directly from the interface. |
| SubnetMask |  | An IP address (Auto generated) | 6/6 | These values shall not be configured. They are retrieved directly from the interface. |

2.1.13 Network.Resolver

Description: Enable /disable retrieval of Domain Name System settings from DHCP server. The actual DNS settings in use by the operating system are located in this group.

[Network.Resolver]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| NameServer1 |  | Auto generated | 6/6 | This parameter is read only. |
| NameServer2 |  | Auto generated | 6/6 | This parameter is read only. |

| ObtainFromDHCP | yes | yes,no | 6/6 | Specifies if the DNS server should be obtained from a DHCP server. |
| --- | --- | --- | --- | --- |
| Search |  | Auto generated | 6/6 | This parameter is read only. |

2.1.14 Network.Routing

Description: Routing table actually in use by the operating system. [Network.Routing]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| DefaultRouter |  | Auto generated | 6/6 | This parameter is read only. |

2.1.15 Network.RTP

| Description: Parameters related to multicast RTP. [Network.RTP] Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| NbrOfRTPGroups | Hardware dependent | 1 ... | 6/6 | The number of RTP groups. One group for each possible multicast presentation (i.e. video source). |
| StartPort | 50000 | 1024 ... 65532 | 6/6 | The RTP port range defines the range of ports from which the video/audio ports are automatically selected. This feature is useful if the product is connected to a NAT router with manually configured port mapping. Note: Each RTP session needs |

|  |  |  |  | 4 ports, which means 4 ports for each unicast session (audio and video) or 4 ports for the multicast session in total. |
| --- | --- | --- | --- | --- |
| EndPort | 50999 | 1025 ... 65535 | 6/6 | The RTP port range defines the range of ports from which the video/audio ports are automatically selected. This feature is useful if the product is connected to a NAT router with manually configured port mapping. Note: Each RTP session needs 4 ports, which means 4 ports for each unicast session (audio and video) or 4 ports for the multicast session in total. |

2.1.16 Network.RTSP

Description: Parameters needed by the RTSP daemon. [Network.RTSP]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Enabled | yes | yes,no | 6/6 | Enable/disable RTSP support. If disabled, only multicast RTP is available for MPEG delivery. |
| Port | 554 | 554, 1024 ... 65535 | 6/6 | The port number for the RTSP daemon. |

2.1.17 Network.UPnP

Description: Enable/disable Universal Plug and Play and set the name to be displayed in UPnP-clients.

[Network.UPnP]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Enabled | yes | yes,no | 6/6 | Enables Universal Plug and Play. |
| FriendlyName | <product name> - <MAC> | A string | 6/6 | The name of the UPnP device. |

2.1.18 Network.UPnP.NATTraversal

Description: These parameters control NAT traversal functionality. NAT traversal is a technique that can be used to open up routers and firewalls to make devices on a LAN accessible from the Internet.

[Network.UPnP.NATTraversal]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Enabled | no | yes,no | 6/6 | Enables/disables NAT traversal. |
| Router |  | An IP address | 6/6 | If an IP address is entered NAT traversal will be attempted with that router. If none is entered, the server will automatically try to discover a router. |
| ExternalIPAddress |  | An IP address | 6/6 | The external IP address of the NAT router. This value shall not be configured, it is set by the system itself. |
| Active | no | yes,no | 6/6 | This parameter is set to yes if NAT traversal was successful. This value shall not be configured, it is set by the system itself. |
| MinPort | 32768 | 1 ... 65535 | 6/6 | The first time NAT traversal is enabled, a random port between MinPort and MaxPort will be selected as the TCP port to map in |

|  |  |  |  | the router. If port mapping is successful, that port will be used thereafter. You can limit the random range by setting MinPort and MaxPort. |
| --- | --- | --- | --- | --- |
| MaxPort | 65535 | 1 ... 65535 | 6/6 | The first time NAT traversal is enabled, a random port between MinPort and MaxPort will be selected as the TCP port to map in the router. If port mapping is successful, that port will be used thereafter. You can limit the random range by setting MinPort and MaxPort. |

2.1.19 Network.ZeroConf

Description: Enable /disable automatic configuration of link local IP address. The negotiated network settings are located in this group, and are used in parallel with the setting of the Network.eth0 group.

I.e. both addresses can be used simultaneously.

[Network.ZeroConf]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Enabled | no | yes,no | 6/6 | Enable/disable zero configuration. |
| IPAddress |  | Auto generated | 6/6 | This parameter is read only. |
| SubnetMask |  | Auto generated | 6/6 | This parameter is read only. |

2.1.20 NetworkLED

Description: This parameter group controls the behavior of the network LED. [NetworkLED]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Usage | on | on,off | 6/6 | Network LED behavior. |

2.1.21 SMTP

Description: Parameters for the Simple Mail Transfer Protocol, for sending e-mail messages between mail servers.

[SMTP]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| FromEmail |  | An e-mail address | 6/6 | E-mail address of the individual or device from which the e-mail is sent. |
| MailServer1 |  | An IP address or a host name | 6/6 | Primary mail server. |
| Port | 25 | 1~65535 | 6/6 | A Port of SMTP server |
| ToEmail |  | An e-mail address | 6/6 | E-mail address of the individual which receiver the e-mail. |
| Subject |  | Subject | 6/6 | Mail subject. |

2.1.22 SMTP.Authentication.A#

| Description: Parameters for SMTP authentication. [SMTP.Authentication.A0] Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Enabled | no | yes,no | 6/6 | Enable/disable authentication. |
| UserName |  | A string | 6/6 | The user name for |

|  |  |  |  | the mail server or the POP server. |
| --- | --- | --- | --- | --- |
| Password |  | A string | 6/6 | The password for the mail server or the POP server. |
| WeakestMethod | Login | Login,Plain,CRAM-MD5 | 6/6 | The weakest method allowed for mail server login. |
| UsePOP | no | yes,no | 6/6 | Login to a POP server. |
| POPServer |  | An IP address or a host name | 6/6 | The IP address or host name of the POP server. |

*Note: The # is replaced with a group number 1 or 2. The number is representing mail server 1 and mail server

2.1.23 StatusLED

Description: This group controls the behavior of the status LED. [StatusLED]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Usage | on | on,off | 6/6 | Status LED behavior during normal operation. |

2.1.24 System

| Description: Parameters for HTTP access to the device. [System] Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| BoaPort | 80 | 1 ... 65535 | 6/6 | HTTP port number. |

| AlternateBoaPort | 80 | 0 ... 65535 | 6/6 | Secondary HTTP port number. Disable by setting it to zero. This parameter is set automatically when NAT traversal is enabled. |
| --- | --- | --- | --- | --- |
| BoaProtViewer | password | password,anonymous | 6/6 | password = Password is required to access the view pages. anonymous = Anybody on the network can access the product's view pages (but not the Admin tools) in a browser, and without having to log in. |
| HTTPPort | 80 | 80 | 6/6 | HTTP port number. |
| HTTPViewerPort | 2000 | 2000 | 6/6 | HTTP port number for viewer/recorder API. |

2.1.25 Time

Description: Common time information which tell the time zone, how date and time is synchronized and the offset related to the chosen time zone and Coordinated Universal Time, UTC.

[Time]

| Parameter name | Default value | Valid values | Securi ty level (get/s et) | Descriptio n |
| --- | --- | --- | --- | --- |

| ObtainFrom DHCP | yes | yes,no | 6/6 | DHCP servers may provide names/IP addresses for local/remo te NTP servers. Enable this feature by setting this parameter to yes. |
| --- | --- | --- | --- | --- |
| ServerDate |  | A date | 6/6 | The date (yyyy-mm -dd) when the device's time was set manually or synchroniz ed with the computer. |
| ServerTime |  | A time | 6/6 | The time (hh:mm:s s) when the device's time was set manually |

|  |  |  |  | or synchroniz ed with the computer. |
| --- | --- | --- | --- | --- |
| SyncSource | Product depend ent | PC,NTP,None | 6/6 | The source to synchroniz e the time with; PC, NTP or None (manually ). |
| TimeZone | GMT | GMT-1200,...GMT-1000,GMT,GMT+100 0,...GMT+1200 | 6/6 | Time zone. |
| UTCOffset | 00:00: 00 | A time | 6/6 | Time difference (hh:mm:s s) between the time zone and Coordinat ed Universal Time (UTC). -01:00:00 = -1 hour, i.e. 1 hour after UTC. 00:00:00 = 0 hour, 01:00:00 = 1 hour, i.e. before UTC. |

2.1.26 Time.DST

| Description: Contain parameters required to manage Daylight Saving Time, DST. [Time.DST] Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Enabled | no | yes,no | 6/6 | Enable/disable DST (Daylight Saving Time) |
| Offset | 01:00:00 | A time | 6/6 | The amount of time the clock should be turned back/forward (hh:mm:ss), due to DST. |
| StartDay | 0 | 1 ... 31, 0 ... 6 | 6/6 | The meaning of StartDay depends on StartTypeOfDate. If StartTypeOfDate is 0 (exact date) the StartDay should be interpreted as the day of the month. Otherwise StartDay indicates the number of days since Sunday in the range 0 to 6. |
| StartMonth | 0 | 0 ... 11 | 6/6 | The number of months since January in the range 0 to 11. |
| StartTime | 00:00:00 | A time | 6/6 | Indicates the time (hh:mm:ss) when DST should be enabled. StartTime = 02:00:00 means that DST should be enabled two hours after midnight. |
| StartTypeOfDate | 0 | -1,0,1 ... 31 | 6/6 | DST can either start on an exact date, or a specific weekday of the month. StartTypeOfDate indicates how to interpret StartDay. If 0, then StartDay is an exact date, otherwise it is a weekday. 0 = StartDay is an exact date (1-31). |

|  |  |  |  | -1 = The weekday specified by StartDay is the last in the month. 1 to 31 = The first weekday specified by StartDay that has a date larger than or equal to this. Example: StartTypeOfDate = 0, StartDay = 12, The 12th of the month, Example2: StartTypeOfDate = -1, StartDay = 0, The last Sunday of the month, Example 3: StartTypeOfDate = 1, StartDay = 5, The first Friday of the month, Example 4: StartTypeOfDate = 15, StartDay = 0, The third Sunday of the month |
| --- | --- | --- | --- | --- |
| StopDay | 0 | 1 ... 31, 0 ... 6 | 6/6 | The meaning of StopDay depends on StopTypeOfDate. If StopTypeOfDate is 0 (exact date) then StopDay should be interpreted as the day of the month. Otherwise StopDay indicates the number of days since Sunday in the range 0 to 6. |
| StopMonth | 0 | 0 ... 11 | 6/6 | The number of months since January in the range 0 to 11. |
| StopTime | 00:00:00 | A time | 6/6 | Indicates the time (hh:mm:ss) when DST should be disabled. StopTime = 02:00:00 means that DST should be disabled two hours after midnight. |
| StopTypeOfDate | 0 | -1 0 1 ... 31 | 6/6 | DST can either end on an exact date, or a specific weekday of the month. See the description of StartTypeOfDate above. |

2.1.27 Time.NTP

Description: Contain parameters required when setting time and date with the NTP

protocol.

[Time.NTP]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Server | 0.0.0.0 | An IP address or a host name | 6/6 | The NTP server to connect to when synchronizing the time in the device. |
| Update | 01:00:00 | A time | 6/6 | Time interval (hh:mm:ss) between connections to the NTP server. |
| VolatileServer |  | An IP address or a host name | 6/6 | The name/IP address of the NTP server, received from the DHCP server. Only one NTP server is currently supported. The NTP server name/IP address will be valid only until the next DHCP renewal or reboot. |

2.1.28 Network.PPPoE

Description: Network settings of the first ethernet interface. Use these parameters to retrieve the network settings actually in use by the operating system.

[Network.PPPoE]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Enabled | no | yes,no | 6/6 | Enable/disable PPPoE dial function |
| Username |  | a string username | 6/6 | username needed to login the PPPoE server |

| Password |  | a string password | 6/6 | password needed to login the PPPoE server |
| --- | --- | --- | --- | --- |
| IPAddress |  | An IP address (Auto generated) | 6/6 | IP Address. The physical address of the device on the network. This parameter is read only. |
| SubnetMask |  | An IP address (Auto generated) | 6/6 | Subnet mask. Divides the network. This parameter is read only. |
| DefaultRouter |  | An IP address (Auto generated) | 6/6 | Default Router Address. This parameter is read only. |
| NameServer1 |  | An IP address (Auto generated) | 6/6 | DNS get from PPPoE server. |
| Status | Disconnect | Connect, Disconnect, Connect Fail | 6/6 | PPPoE connect status. |

2.1.29 Network.DIPS

Description: Dynamic IP service

[Network.DIPS]

| Parameter name | Default value | Valid value s | Security level (get/set ) | Description |
| --- | --- | --- | --- | --- |
| RigisterPath | http://www.airlive.com/register.ph p | a string | 6/6 | Register path of dynamic IP services |
| RequestPat h | http://www.airlive.com/request.ph p | a string | 6/6 | Request path dynamic IP services |
| Enabled | no | yes, no | 6/6 | Enable/disabl e dynamic IP |

|  |  |  |  | services |
| --- | --- | --- | --- | --- |
| ID |  | a string | 6/6 | Device ID for DIPS, generate automatically. |

2.1.30 Network.SIP

Description: SIP service

[Network.SIP]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Enabled | no | yes, no | 6/6 | Enable/disable SIP services |
| Username |  | a string username | 6/6 | username needed to login the SIP server |
| Password |  | a string password | 6/6 | password needed to login the SIP server |
| Domain | 0.0.0.0 | An IP address or a host name | 6/6 | The SIP server. |

2.2 JPEG/MJPEG/MPEG

2.2.1 Image

| Description: Common image parameters used for all image configurations. [Image] Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| DateFormat | YYYY-MM-DD | Www Mmm DD YYYY, YYYY-MM-DD, MM/DD/YYYY | 6/6 | OSD Date format |

| OwnDateFormatEnabled | no | yes, no | 6/6 | User define date format |
| --- | --- | --- | --- | --- |
| OwnDateFormat | %Y-%m-%d |  | 6/6 | Date format |
| TimeFormat | 24 | 12, 24 | 6/6 | OSD Time format |
| OwnTimeFormatEnabled | no | yes, no | 6/6 | User define time format |
| OwnTimeFormat | %H:%M:%S |  | 6/6 | Time format |
| MaxViewers | 10(mpeg4), 20(h264) | 1~10(mpeg4), 1~20(h264) | 6/6 | Max number of simultaneous viewers (does not affect multicast delivery). |

• OwnDateFormat and OwnTimeFormat could refer following description

•	%a	The abbreviated weekday name

according to the   current locale.

•

•	%A	The full weekday name according to the

current locale.

•

•	%b	The abbreviated month name according

to the   current locale.

•

•	%B	The full month name according to the

current locale.

•

•	%c	The preferred date and  time

representation for the   current locale.

•

•	%C	The century number (year/100) as  a

2-digit integer.

•

•	%d	The day  of  the   month as  a  decimal

number (range 01  to 31).

•

•	%D	Equivalent to %m/%d/%y. (Yecch -- for

Americans 	only.	Americans 	should	note 	that 	in other	countries

•		%d/%m/%y is rather common. This  means that  in international  context this  format is  ambiguous

and should not

•	be used.)

•

•	%e	Like  %d, the   day  of  the   month as  a

decimal number, but  a leading zero  is replaced by a space.

•

•	%E	Modifier: use alternative format, see

below.

•

•	%F	Equivalent to %Y-%m-%d  (the ISO 8601

date format).

•

•		%G		The ISO 8601  year   with  century as  a decimal  number. 	The 4-digit year  corresponding to the  ISO  week	number

• 		(see %V).	This  has  the   same  format and value as %y, except that if the  ISO week number belongs

to the   pre-

• 	vious or  next  year, that year  is used

instead.

•

•	%g	Like  %G,  but  without century,  i.e.,

with  a  2-digit year   (00-99).

•

•	%h	Equivalent to %b.

•

•	%H	The hour  as  a decimal number using a

24-hour clock (range  00  to 23).

•

•	%I	The hour  as  a decimal number using a

12-hour clock (range  01  to 12).

•

•	%j	The day of the  year  as a decimal number

(range 001  to 366).

•

•		%k	The hour  (24-hour clock) as  a decimal number (range 0 to 23); single digits are  preceded by a

blank. 	(See

• 	also %H.)

•

•		%l	The	hour 	(12-hour clock) as  a decimal number (range 1 to 12); single digits are  preceded

by a  blank.  (See

• 	also %I.)

•

•	%m	The month as  a decimal number (range

01 to 12).

•

•	%M	The minute  as  a decimal number (range

00 to 59).

•

•	%O	Modifier: use alternative format, see

below.

•

•		%p	Either 'AM' or  'PM'  according to the  given  time  value, or  the  corresponding strings for the

current locale.

•	Noon is treated as  'pm' and midnight

as  'am'.

•

•	%P	Like %p but  in lowercase: 'am' or 'pm'

or  a  corresponding string  for  the   current locale.

•

•	%r	The time in a.m.  or p.m. notation. 	In

the  POSIX  locale this  is  equivalent to '%I:%M:%S  %p'.

•

•	%R	The time  in 24-hour notation (%H:%M).

For  a  version including the   seconds, see  %T  below.

•

•	%s	The number of seconds since the  Epoch,

i.e., since  1970-01-01 00:00:00 UTC.

•

•	%S	The	second  as  a  decimal number

(range 00 to 60).	(The  range   is up to 60 to allow  for occasional leap sec-

• 	onds.)

•

•	%T	The time  in 24-hour notation

(%H:%M:%S).

•

•	%u	The day of the  week as a decimal, range

1 to 7, Monday being   1.	See  also %w.

•

•		%U	The week number of the  current year  as a decimal number,  range  00 to 53,  starting with  the  first

Sunday	as

•	the  first day of  week 01.  See also %V

and %W.

•

•		%V	The	ISO 8601:1988  week number of  the current year  as  a decimal number,  range  01 to 53,  where week 1  is  the

•		first week that has at least 4 days  in the current year, and with  Monday as  the  first day of  the

week. 	See

• 	also %U   and  %W.

•

•	%w	The day of the  week as a decimal, range

0 to 6, Sunday  being   0.	See  also %u.

•

•		%W	The	week number of  the  current year  as  a  decimal number,  range  00 to 53,  starting with  the

first Monday as

•	the  first day  of  week 01.

•

•	%x	The preferred date representation for

the  current locale  without the   time.

•

•	%X	The preferred time representation for

the  current locale  without the   date.

•

•	%y	The year  as  a decimal number without

a century (range 00  to 99).

•

•	%Y	The year  as a decimal number including

the  century.

•

•	%z	The time-zone as hour offset from GMT.

Required to emit  RFC 822-conformant dates 	(using 	"%a,

%d 	%b 	%Y

•	%H:%M:%S  %z").

•

•	%Z	The time zone or name or abbreviation.

•

•	%%	A   literal '%'   character.

•

2.2.2 Image.I0

Description: Image configuration. Contains the name of the image configuration and which image source it is connected to.

[Image.I0]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Name | 1 | A string | 6/6 | Name of the sourc. |

*Note: only support one source.

2.2.3 Image.I0.Appearance

Description: Image appearance parameters (resolution) for each image configuration.

[Image.I0.Appearance#] **MPEG4 platform

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Resolution |  | 1280x1024, | 6/0 | The image resolution. |

|  |  | 1024x768, VGA, CIF, QCIF |  |  |
| --- | --- | --- | --- | --- |
| Compression | 90 | 0 ... 100 | 6/0 | The level of image compression. High compression reduces the file size. Low compression produces optimum picture quality, but larger file sizes. MPEG-4: The compression setting maps to the qscale used when quantizing the video stream. This mapping has been made in such a way as to give as similar result as using the same setting for M-JPEG. |
| Enabled | yes | yes,no | 6/0 | Profile enabled : multi-profile mode |
| Format |  | mjpeg, h264, mpeg4 | 6/0 | Each format only support three proflie. |

Appearance#:

|  | CMOS Mega | CMOS VGA | CCD VGA |
| --- | --- | --- | --- |
| Image.I0.Appearance | mjpeg, 1280x1024 | mjpeg, CIF | mjpeg, CIF |
| Image.I0.Appearance1 |  | mpeg4, VGA | mpeg4, VGA |
| Image.I0.Appearance2 |  | mpeg4, CIF | mpeg4, CIF |
| Image.I0.Appearance3 |  | mpeg4, QCIF | mpeg4, QCIF |
| Image.I0.Appearance4 |  | mjpeg, CIF | mjpeg, CIF |
| Image.I0.Appearance5 | mjpeg, 1280x1024 |  |  |
| Image.I0.Appearance6 | mpeg4, 1024x768 |  |  |

[Image.I0.Appearance#] **H264 platform

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Resolution |  | SXGA, VGA, | 6/0 | The image resolution. |

|  |  | QVGA, QQVGA, 4CIF, CIF, QCIF |  |  |
| --- | --- | --- | --- | --- |
| Compression | 90 | 0 ... 100 | 6/0 | The level of image compression. High compression reduces the file size. Low compression produces optimum picture quality, but larger file sizes. MPEG-4: The compression setting maps to the qscale used when quantizing the video stream. This mapping has been made in such a way as to give as similar result as using the same setting for M-JPEG. |
| Enabled | yes | yes,no | 6/0 | Profile enabled : multi-profile mode |
| Format |  | mjpeg, h264, mpeg4 | 6/0 | Each format only support three proflie. |
| Type | Normal | Normal, ROI | 6/0 | Specified the profile is for ROI or not. |
| Name |  | ROI0, ROI1 | 6/0 | Specified the ROI name. |
| Position | 0x0 |  | 6/0 | Specified the position of ROI window. |

Appearance#:

|  | CMOS Mega | CMOS VGA | CCD 4CIF |
| --- | --- | --- | --- |
| Image.I0.Appearance | mjpeg, vga | mjpeg, vga | mjpeg, 4cif |
| Image.I0.Appearance1 | h264, sxga | h264, vga | h264, 4cif |
| Image.I0.Appearance2 | h264, vga | h264, qvga | h264, cif |
| Image.I0.Appearance3 | h264, qvga | h264, qqvga | h264, qcif |
| Image.I0.Appearance4 | h264, qqvga | mpeg4, vga | mpeg4, 4cif |
| Image.I0.Appearance5 | mpeg4, vga | mpeg4, qvga | mpeg4, vcif |
| Image.I0.Appearance6 | mpeg4, qvga | mpeg4, qqvga | mpeg4, qcif |
| Image.I0.Appearance7 | mpeg4, qqvga | mjpeg, vga | mjpeg, 4cif |

| Image.I0.Appearance8 | mjpeg, sxga | mjpeg, qvga | mjpeg, cif |
| --- | --- | --- | --- |
| Image.I0.Appearance9 | mjpeg, vga | mjpeg, qqvga | mjpeg, qcif |
| Image.I0.Appearance10 | mjpeg, qvga |  |  |
| Image.I0.Appearance11 | mjpeg, qqvga |  |  |

2.2.4 Image.I0.MPEG

Description: Parameters related to the generation and format of the MPEG stream. [Image.I0.MPEG]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| ICount | 1 | 1 | 6/6 | The number of I frames per GOV |
| PCount | 60 | 1~60 | 6/6 | The number of P frames per I frame |

2.2.5 Image.I0.Overlay

| Description: Image overlay parameters for each image configuration. [Image.I0.Overlay] * Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Enabled | no | yes,no | 6/6 | Whether to show the overlay image or not. The overlay image is stored in a file given by the parameter Image.OverlayPath. |
| XPos | 0 | An unsigned integer | 6/6 | The overlays upper left corner is positioned at this horizontal position (0 = to the left). |
| YPos | 0 | An unsigned integer | 6/6 | The overlays upper left corner is positioned at this horizontal position (0 = to the top). |

*Note: The # is replaced with a group number starting from zero, e.g. Image.I0.Overlay.

2.2.6 Image.I0.Overlay.MaskWindows

Description: Privacy mask parameters for each image configuration. [Image.I0.Overlay.MaskWindows] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Color | black | black,grey,white,red | 6/6 | The privacy mask color. |

2.2.7 Image.I0.Overlay.MaskWindows.M#

Description: Privacy mask parameters for each image configuration. [Image.I0.Overlay.MaskWindows.M#] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Enabled | no | yes,no | 6/6 | Enable/disable privacy mask. |
| XPos | 0 | An unsigned integer | 6/6 | The overlays upper left corner is positioned at this horizontal position (0=to the left). |
| YPos | 0 | An unsigned integer | 6/6 | The overlays upper left corner is positioned at this vertical position (0=to the top). |
| Width | 0 | An unsigned integer | 6/6 | Width of the specified mask. |
| Height | 0 | An unsigned integer | 6/6 | Height of the specified mask. |

*Note: The # is replaced with a group number starting from zero, e.g. Image.I0.Overlay.MaskWindows.M0

2.2.8 Image.I0.RateControl

| Description: Parameters to control the bit rate (bandwidth) from the server. [Image.I0.RateControl#] * Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Mode | vbr | vbr,cbr | 6/6 | Specifies whether the rate controller operates in Variable Bit Rate (VBR) or constant bit rate (CBR) mode. |
| TargetBitrate | 384 | 384 ... 2048 Mpeg4 VGA 128 ... 1024 Mpeg4 QVGA 24 ...320 Mpeg4 QQVGA 128 ... 1024 JPEG QVGA | 6/6 | The target bit rate, in kbit/s, for CBR mode. |
| MaxFps | 30 | 1 ... 30 | 6/6 | The rate controller will not produce streams with a frame rate higher than this value. Note 1: The parameter Image.I0.RateControl.Mode must be set to cbr for this parameter to take effect. Note 2: If also setting the parameter Image.I0.Stream.FPS to a value, the lowest value will apply |

|  |  |  |  | for the MPEG-4 stream. |
| --- | --- | --- | --- | --- |
| MaxCompression | 100 | 0 ... 100 | 6/6 | The rate controller will not produce streams with compression higher than this value. |
| MinCompression | 0 | 0 ... 100 | 6/6 | The rate controller will not produce streams with compression lower than this value. Note: Image.I0.Appearance.Compression overrides this value. |

*Note: The # is replaced with a profile number, e.g. Image.I0.RateControl for profile0, Image.I0.RateControl1 for profile1

2.2.9 Image.I0.Stream

Description: Image streaming parameters for each image configuration. [Image.I0.Stream#] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| FPS | 30 | VGA NTSC : 30, 20, 15, 10, 7, 6, 5, 4, 3, 2, 1 VGA PAL : 25, 20, 12, 6, 5, 4, 3, 2, 1 MEGA MPEG4 : 15, 10, 7, 5, 4, 3, 2, 1 MEGA MJPEG : 5, 4, 3, 2, 1 | 6/6 | Limits the frame rate available to each viewer. |

*Note: The # is replaced with a profile number, e.g. Image.I0.Stream.FPS for stream of profile0, Image.I0.Stream1.FPS for stream of profile1

2.2.10 Image.I0.Text

Description: Image text overlay parameters for each image configuration.

[Image.I0.Text] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| BGColor | black | white, black, transparent, semitransparent | 6/6 | Text background color. |
| ClockEnabled | no | yes,no | 6/6 | Shows the time at the Position in the image. |
| Color | white | white, black | 6/6 | Text color. |
| DateEnabled | no | yes,no | 6/6 | Shows the date at the Position in the image. |
| Position | top | top,bottom | 6/6 | Positions text at top or bottom of the image. |
| String |  | A string | 6/6 | The text to show at the Position in the image. |
| TextEnabled | no | yes,no | 6/6 | Shows the String at the Position in the image. |

* Note: The # is replaced with a group number starting from zero, e.g. Image.I0.Text.

2.2.11 ImageSource

| Description: The number of image sources (video inputs/chips or CCD/CMOS chips). [ImageSource] Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| NbrOfSources | 1 in | An unsigned | 6/6 | Number of image |

|  | one-input products | integer (Read only) |  | sources/inputs. |
| --- | --- | --- | --- | --- |

2.2.12 ImageSource.I0

Description: The name of the image source, only support on image source. [ImageSource.I0] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Name | Video in one-input video products. | A string | 6/6 | Name of the image source/input. |

2.2.13 ImageSource.I0.DayNight

Description: Parameters related to the behaviour of the IR cut filter. This parameter group is product dependent and only available in some network cameras.

[ImageSource.I0.DayNight]

1.ICR Control

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| ICR | day | day, night, auto, schedule | 6/6 | The behaviour of the IR cut filter. day = IR filter always enabled. night = IR filter always disabled. auto = IR filter removed automatically. |
| IcrOnThreshold | 3800 | 0 ~ 10000 | 6/6 | ICR on threshold. While the gain is lower than the threshold, icr will be enaled. (night mode) |
| IcrOffThreshold | 2000 | 0 ~ 10000 | 6/6 | ICR off threshold. While the gain is higher than the threshold, icr will be disaled. (day mode) |
| IcrEffectTime | 5 | 0 ~ 85536 | 6/6 | ICR triggered duration. |

2.IR Control

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| IrCutFilter | no | yes, no, schedule, auto | 6/6 | The behaviour of the IR cut filter. yes = IR filter always enabled. no = IR filter always disabled. schedule = IR filter removed by schedule. |
| IROnThreshold | 2000 | 0 ~ 10000 | 6/6 | IR on threshold. While the gain is higher than the threshold, ir will be enaled. (night mode) |
| IROffThreshold | 3800 | 0 ~ 10000 | 6/6 | IR off threshold. While the gain is lower than the threshold, ir will be disaled. (day mode) |
| IREffectTime | 5 | 0 ~ 85536 | 6/6 | IR triggered duration. |

3.Image Control (B/W mode)

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| ImgCtrl | color | bw, color, auto, schedule | 6/6 | The behaviour of the image control. color = image is in color mode. bw = image is in B/W mode. auto = image mode change automatically. schedule = changed by event schedule |
| ImgCtrlBWThreshold | 2000 | 0 ~ 10000 | 6/6 | Image color mode threshold. While the gain is lower than the threshold, image will change to color mode. |
| ImgCtrlColorThreshold | 3800 | 0 ~ 10000 | 6/6 | Image B/W mode threshold. While the gain is |

|  |  |  |  | higher than the threshold, image will change to B/W mode. |
| --- | --- | --- | --- | --- |
| ImgCtrlEffectTime | 5 | 0 ~ 85536 | 6/6 | ICR triggered duration. |

2.2.14 ImageSource.I0.Sensor

Description: Parameters for each CCD/CMOS image source. This parameter group is product dependent and only available in network cameras. Check the product specification for supported parameters, default values and valid values.

[ImageSource.I0.Sensor]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| ColorLevel | 50 | 0 ... 100 | 6/6 | The image color level. |
| Brightness | 50 | 0 ... 100 | 6/6 | The image brightness. |
| Contrast | 50 | 0 ... 100 | 6/6 | The image contrast. |
| Sharpness | Product dependent | Product dependent | 6/6 | The image sharpening. |
| Exposure | auto | auto,hold | 6/6 | The image exposure. Note: Valid values are product dependant. |
| NightVision | off | on,off | 6/6 | Will increase sensitivity in low light environments when on. |
| MaxExposure | 4 | 1,2,4,8,12,24 | 6/6 | Max exposure time : 1/120 S, 2/120 S, 4/120 S, 8/120 S, 12/120 S, 24/120 S |
| WhiteBalance | auto | auto,hold | 6/6 | The image white balance. |
| AntiBlooming | no | yes, no | 6/6 | The image anti-blooming. (cmos sensor only) |

| LocalOutput | no | yes, no | 6/6 | Local output image. (ccd sensor only) |
| --- | --- | --- | --- | --- |
| WDR | yes | yes, no | 6/6 | Hight Dynamic Range mode |
| BLC | on | on, off | 6/6 | Backlight comprtition |
| D-Brightness | auto | off, auto | 6/6 | D-Brightness |
| LowLuxEnhance | auto | off, auto | 6/6 | LowLuxEnhance |

2.2.15 ImageSource.I0.Video

Description: Parameters for each video image source. This parameter group is product dependent. Check the product specification for supported parameters, default values and valid values.

[ImageSource.I0.Video] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| DetectedType | none | none,pal,ntsc | 6/6 | Which type of video source that is connected. |

2.3 Guard Tour

2.3.1 GuardTour.G#

Description: Contains parameters to create PTZ guard tours. Maximum support 16 guardtours.

[GuardTour.G#] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Running | no | yes,no | 6/6 | Enable/disable the guard tour. |
| Name |  | A string, GuardtourName | 6/6 | The name of the guard tour. |

* Note: The # is replaced with a group number starting from zero, e.g. GuardTour.G0.

2.3.2 GuardTour.G#.Tour.T#

Description: The PTZ preset positions that are included in the guard tour. [GuardTour.G#.Tour.T#] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| PresetNbr | 0 | An unsigned integer | 6/6 | The number of the PTZ preset position. |
| Position | 1 | An unsigned integer | 6/6 | In which order to view this preset position. |
| Movespeed | 70 | An unsigned integer | 6/6 | The speed at which to move the camera to this preset position. |
| WaitTime | 10 | An unsigned integer | 6/6 | The view time for this preset position in seconds or minutes. |
| WaitTimeViewType | Seconds | Seconds,Minutes | 6/6 | The time unit for the WaitTime. |

* Note: The # is replaced with a group number starting from zero, e.g. GuardTour.G0.Tour.T0.

2.3.3 GuardAutoPan

| Description: Contains parameters to create PTZ autopan. [GuardAutoPan] Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Running | no | yes,no | 6/6 | Enable/disable the guard autopan. |

2.4 PTZ

Note! Unless otherwise noted, write able parameters may not be directly modified by for instance CGI calls to param.cgi. Instead parameter modification is achieved implicitly by various PTZ CGI calls. For instance, PTZ.PTZDrivers.Driver# is set by the PTZ framework when a driver is installed. Setting it directly however, will cause inconsistencies.

2.4.1 PTZ

Description: Various Pan Tilt Zoom parameters. All write able parameters may be directly modified.

[PTZ]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| BoaProtPTZOperator | password | password | 6/6 | ptz control need authentication |
| NbrOfSerPorts | 1 | 1 | 6/6 | Number of camera input. |

2.4.2 PTZ.PTZDrivers

| Description: List of installed PTZ drivers. [PTZ.PTZDrivers] Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Driver0 | none | none | 6/6 | The name of the driver. |
| Driver1 | Pelco-D | Pelco-D | 6/6 | The name of the driver. |
| Driver2 | Pelco-P | Pelco-P | 6/6 | The name of the driver. |
| Driver3 | AirLive | AirLive | 6/6 | The name of the driver. |

2.4.3 PTZ.SerDrivers

Description: List of PTZ drivers associated with serial ports. [PTZ.SerDrivers]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Ser0Driver | none | none, Pelco-D, Pelco-P, AirLive | 6/6 | PTZ drivers associated with camera input |

2.4.4 PTZ.Various

Description: These groups are created dynamically when a driver is installed, based on the configuration file of the driver. '#' denotes the index for the driver when installed. The parameters here are only used as default values for other parameter groups.

[PTZ.Various.V0]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| CamId | -1 | An integer | 0/6 | Identifies a PTZ unit (camera) on a serial port, often set by a dip switch on the unit. |
| OSDMenuOpen |  | String | 0/6 | OSD menu command string |
| OSDMenuClose |  | String | 0/6 | OSD menu command string |
| OSDMenuSelect |  | String | 0/6 | OSD menu command string |
| OSDMenuBack |  | String | 0/6 | OSD menu command string |
| OSDMenuUp |  | String | 0/6 | OSD menu command string |
| OSDMenuRight |  | String | 0/6 | OSD menu command string |
| OSDMenuLeft |  | String | 0/6 | OSD menu command string |
| OSDMenuDown |  | String | 0/6 | OSD menu command string |

2.4.5 PTZ.PresetPos

A dynamic parameter group PTZ.PresetPos.P# is created for each new preset position. # merely denotes the number of the dynamic parameter group and has no connection to any preset position numbers mentioned below. Maximum support 64 presets. Description: Dynamic parameter groups, each representing a preset position.

[PTZ.PresetPos.P#]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Pos | None | A : 2 delimited string with the following content: <Name>: A valid preset position name according to the PTZ control section in AirLive HTTP API. <Type>: server, device <No>: 1, ... for <Type> = server, -1 for <Type> = device <DevicePresetNo>: An integer <DriverNo>: 1, ... <CamNo>:1, ... <Pan>: -180 ... 180, 10000 <Tilt>: -180 ... 180, 10000 <Zoom>: 1 ... 9999, 10000 <Focus>: 1 ... 9999, 10000 <Iris>: 1 ... 9999, 10000 | 1/6 | Holds all necessary information about the preset position. <Name>: The name of the preset position. <Type>: The type of preset position, "server" for server preset positions, "device" for device preset positions. <No>: The number of the server preset position, -1 for device preset positions. <DevicePresetNo>: The number of the device preset position, not significant if <Type> = "server" and the position is not emulated by a driver preset position. <DriverNo>: The number (index) of the concerned driver. <CamNo>: The number of the concerned camera. <Pan>: The pan position if known, else 10000. <Tilt>: The tilt position if known, else 10000. |

|  |  | <Home>: true, false |  | <Zoom>: The zoom position if known, else 10000. <Focus>: The focus position if known, else 10000. <Iris>: The iris position if known, else 10000. <Home>: true if the preset position is the home position for the camera, else false. Example of a server preset position (number 3) emulated by a device preset position (number 12): Entrance~server:3:12:1: 2:45:-10:10000:10000: 10000:false |
| --- | --- | --- | --- | --- |

1 May be overridden by configuration file for driver.

2 Except for Name, delimited by ~.

2.4.6 PTZ.Preset.P#

| Description: Preset PTZ position configuration. [PTZ.Preset.P#]* Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Name | Hardware dependent | A string | 1/6 | The name of the preset position. |
| ImageSource | Hardware dependent | 0 (in products with 1 ImageSource), 0 ... 4 (in products with 4 ImageSource (4 inputs + Quad) | 1/6 | The ImageSource.I0 this Preset.P# configuration is connected to. |
| HomePosition | 0 | An unsigned integer | 1/6 | The Position.P# group number of the |

|  |  |  |  | home position. |
| --- | --- | --- | --- | --- |

* Note: The # is replaced with a group number starting from zero, e.g. PTZ.Preset.P0

2.4.7 PTZ.Preset.P#.Position.P#

Description: Position and zoom parameters for each preset PTZ position configuration.

[PTZ.Preset.P#.Position.P#]*

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Name | Hardware dependent | A string | 1/6 | The name of the position. |
| Data | Hardware dependent | A string | 1/6 | The pan, tilt and zoom values of the position. |

2.5 Motion Detection

2.5.1 Motion.M#

Description: Contain dynamic parameters, which define the properties of a Motion

| Detection window. [Motion.M#] * Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Name | DefaultWindow | A string | 6/6 | A user's window identification, e.g. "Door" or "Window". |
| ImageSource | 0 | 0 ... 3 | 6/6 | In Which image source the motion detection window is located. |
| Left | 200 | 0 ... 9999 | 6/6 | The coordinate for the left boundary of the |

|  |  |  |  | rectangular motion detection window. The full value range is related to the full image width and 0 is counted from the left hand side of the image. |
| --- | --- | --- | --- | --- |
| Right | 4000 | 0 ... 9999 | 6/6 | The coordinate for the right boundary of the rectangular motion detection window. The full value range is related to the full image width and 0 is counted from the left hand side of the image. |
| Top | 200 | 0 ... 9999 | 6/6 | The coordinate for the upper boundary of the rectangular motion detection window. The full value range is related to the full image height and 0 is counted from the upper side of the image. |
| Bottom | 4000 | 0 ... 9999 | 6/6 | The coordinate for the bottom boundary of the rectangular motion detection window. The full value range is related to the full image height and 0 is counted from the upper side of the image. |
| WindowType | include | exclude,include | 6/6 | Describes the type of the motion detection window. An exclude window, i.e. do not perform motion |

|  |  |  |  | detection in the window. An include window, i.e. perform motion detection in the window area. |
| --- | --- | --- | --- | --- |
| Sensitivity | 90 | 0 ... 100 | 6/6 | This tunes the "object difference from the background"-sensitivity, i.e. difference in terms of color and/or structure. A high value detects even very small changes and can e.g. trigger on image noise if set too high. A very low value requires on the other hand a very dramatic change with e.g. a dark object appearing in an almost white scene (or vice versa). |
| ObjectSize | 15 | 0 ... 100 | 6/6 | Defines the size in percent of the object to result in detection. For a small value, even very small changes trigger the detection while a very large value requires a very large object to trigger the detection. |

* Note: The # is replaced with a group number starting from zero, e.g. Motion.M0.

* Note: Maximum motion window in MPEG4 platform is 4 * Note: Maximum motion window in H264 platform is 10

2.6 I/O

2.6.0 IOPort

Description: New parameter group IOPort, the parameters are taken from the obsolete input.cgi and output.cgi.

[IOPort.I#]

| Parameter | Default value | Valid values | Description |
| --- | --- | --- | --- |
| Configurable | yes | yes, no | New. Check if the port is configurable. Read-only and product dependent. |
| Direction | input | input, output | New. Check if the port is configured as input or output. Read-only for non-configurable ports. |
| Input.Name | Input 1 | string | Moved from Input. |
| Input.Trig | closed | closed, open | Moved from Input. |
| Output.Name | Output 1 | string | Moved from Output. |
| Output.Active | closed | closed, open | Moved from Output. |
| Output.Button | none | none, pulse, actinact | Moved from Output. |
| Output.PulseTime | 0 | integer | Moved from Output. |

2.6.1 Input

Description: Parameters for hardware input(s). [Input]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| NbrOfInputs | Hardware specific | An unsigned integer (Read only) | 0/6 | Number of inputs. |

2.6.2 Input.I#

Description: Parameters for hardware input(s).

[Input.I#] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Name | Input #+1 | A string | 6/6 | The name of the input. |
| ControlSource |  | A string | 6/6 | The control source of the input. |
| Trig | closed | open,closed | 6/6 | Determines when to trig. |

* Note: The # is replaced with a group number starting from zero, e.g. Input.I0.

2.6.3 Output

Description: Parameters for hardware output(s). [Output]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| NbrOfOutputs | Hardware specific | An unsigned integer (Read only) | 0/6 | Number of outputs. |

2.6.4 Output.O#

| Description: Parameters for hardware output(s). [Output.O#] * Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Name | Output #+1 | A string | 6/6 | The name of the output. |
| ControlSource |  | A string | 6/6 | The control source of the input. |
| Active | closed | open, closed | 6/6 | The active state of the output. |

| Button | none | none,pulse,actinact | 6/6 | The button type associated with this output. |
| --- | --- | --- | --- | --- |
| PulseTime | 0 | An integer | 6/6 | The pulsetime for the pulse button connected to this output, if any. |

* Note: The # is replaced with a group number starting from zero, e.g. Output.I0.

2.7 Serial port

Description: Controls the functionality/purpose of the serial ports and some configuration that is shared by various applications.

Note that some applications have their own settings for baudrate, parity etc.

2.7.1 Serial

[Serial]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| NbrOfPorts | 1 | 1 (Read only) | 0/6 | Number of serial ports. |
| PurposePort0 | PTZ | PTZ (Read only) | 6/6 | The current usage of the serial port. PTZ = Pan Tilt Zoom. Supports PTZ control and serial port control according to AirLive HTTP API. Requires an installed PTZ driver. |

2.7.2 Serial.Ser0

[Serial.Ser#] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| BaudRate | 9600 | 300, 600, 1200, 2400, 2400, | 6/6 | The baudrate used in the serial communication. |

|  |  | 4800, 9600, 19200, 38400, 57600, 115200, (for RS485 also 230400 and 460800) |  |  |
| --- | --- | --- | --- | --- |
| DataBits | 8 | 7, 8 | 6/6 | The number of data bits. |
| StopBits | 1 | 1,2 | 6/6 | The number of stop bits. |
| Parity | None | None, Even,Odd | 6/6 | The parity. |

2.8 Events

2.8.1 Event.E#

Description: This group defines an event, which is a set of parameters describing how and when the product performs certain actions. Maximum support 30 events.

[Event.E#]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Name | NewEvent | A string | 6/6 | The name of the event. |
| Type | T | S, T | 6/6 | Scheduled or Triggered event. |
| Enabled | yes | yes, no | 6/6 | Event enabled (disabled events are never triggered). |
| Active | no | yes,no | 6/6 | If the event is currently triggered. |
| HWInputs | xxxx | nnnn,wheren = {x,0,1} | 6/6 | Hardware input trigger pattern. x = do not trigger. 0 = trigger on inactivation. 1 = trigger on activation. Example 1: "1xxx" means trigger when digital input 1 is activated. Example 2: "xx0x" means trigger when digital input 3 is inactivated. |

|  |  |  |  | Example 4: "1xx1" means trigger when digital input 1 and 4 are activated. Example 5: "xxxx" means don't trigger on hardware inputs. Note: Cannot be combined with SWInputs, use either SWInput or HWInputs. |
| --- | --- | --- | --- | --- |
| SWInput |  | A string | 6/6 | Software input trigger pattern. Motion detection: M<motion window no> |
| Weekdays | 1111111 | A string | 6/6 | Pattern of weekdays when the event should trigger. Example 1: "0111110" means trigger on Monday ... Friday. Example 2: "1000000" means trigger on Sunday. |
| Starttime | 00:00 | A time | 6/6 | Time when the event should start to trigger in format "hh:mm".[[BR]]Example 1: "07:30" means trigger at 7:30 am. Example 2: "19:30" means trigger at 7.30 pm. |
| Duration | 00:00 | A time | 6/6 | How long the event should trigger in format "hours:minute". Max 168:00 hours. |

2.8.2 Event FTP Actions

| Description: This group defines an action that uploads files to an FTP server. [Event.E#.Actions.A#] * Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |

| Protocol | FTP | FTP | 6/6 | Protocol. |
| --- | --- | --- | --- | --- |
| Server |  | blank,F0 ... Fn (n = number of FTP event servers - 1) | 6/6 | FTP server ID. Refers to a parameter group under EventServers.FTP. Example: "F0" refers to the parameter group EventServers.FTP.F0. Server2 blank, F0 ... Fn (n = number of FTP event servers - 1) 6/6 Secondary FTP server ID. |

* Note: the # is replaced with a group number, e.g. Event.E0.Actions.A0.

2.8.3 Event HTTP Actions

Description: This group defines an action that uploads files to an HTTP server, or sends notifications to an HTTP server.

[Event.E#.Actions.A#]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Protocol | HTTP | HTTP | 4/4 | Protocol. |
| Server |  | H0 ... Hn (n = number of HTTP event servers -1) | 4/4 | HTTP server ID. Refers to a parameter group under root.EventServers.HTTP. Example: "H0" refers to the parameter group root.EventServers.HTTP.H0. |
| Message |  | A string | 4/4 | Message to send. Possible to use dynamic event notification: #E = The event description. #t = The trigger type and value. #date = event occur date. #time = event occur time. |

| CustomParams |  | A string | 4/4 | Custom parameters to add to URL. Example: "foo=bar". Spaces are not allowed in this field and all text must be URI-encoded (RFC2396). Example: to set the CGI parameter 'example' to 'Y & Z' enter example=Y+%26+Z in this field. |
| --- | --- | --- | --- | --- |

2.8.4 Event HW Actions

Description: This group defines an action that controls a digital output. [Event.E#.Actions.A#] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Protocol | HW | HW | 6/6 | Protocol. |
| Output | 1 | 1 ... 4 | 6/6 | Output number to activate. |
| Duration | 0 | An unsigned integer | 6/6 | How long the output should be active in seconds, 0 means that the output should be active for as long as the event is triggered. -1 means that the output should be active forever. |

* Note: the # is replaced with a group number, e.g. Event.E0.Actions.A0. * Note:

current duration=0 means no action

2.8.5 Event SMTP Actions

Description: This group defines an action that sends mail to a mail server. [Event.E#.Actions.A#] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Protocol | SMTP | SMTP | 6/6 | Protocol. |
| EmailTo |  | A string | 6/6 | E-mail address. |
| Subject |  | A string | 6/6 | Subject line. |
| Message |  | A string | 6/6 | Message body. |

2.8.6 Event TCP Actions

Description: This group defines an action that sends notifications to a TCP/IP server. [Event.E#.Actions.A#] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Protocol | TCP | TCP | 4/4 | Protocol. |
| Server |  | T0 ... Tn (n = number of TCP event servers -1) | 4/4 | TCP server ID. Refers to a parameter group under root.EventServers.TCP. Example: "T0" refers to the parameter group root.EventServers.TCP.T0. |
| Message |  | A string | 4/4 | Message to send. Possible to use dynamic event notification: #E = The event description. #t = The trigger type and value. #date = event occur date. #time = event occur time. |

2.8.7 Event PTZ Actions

Description: This group defines an action that makes a camera go to a preset PTZ

position. [Event.E#.Actions.A#] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Protocol | PTZ | PTZ | 6/6 | Protocol. |
| Preset |  | P0 ... Pn (n = max number of preset positions - 1) | 6/6 | Which PTZ.PresetPos.P# to move to. |

* Note: the # is replaced with a group number, e.g. Event.E0.Actions.A0.

2.8.8 Event Guard Tour Actions

Description: This group defines an action that runs a Guard Tour. [Event.E#.Actions.A#] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Protocol | GUARDTOUR | GUARDTOUR | 6/6 | Protocol. |
| Name |  | A string | 6/6 | Which GuardTour.G# to move to. |

* Note: the # is replaced with a group number, e.g. Event.E0.Actions.A0.

2.8.9 Event IR Actions

| Description: This group defines an action that open the IR light. [Event.E#.Actions.A#] * Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Protocol | IR | IR | 6/6 | Protocol. |
| Duration | 0 | An unsigned integer | 6/6 | How long the output should be active in seconds, 0 means that the output should be active for as long as the event is triggered. -1 means that the output should be |

|  |  |  |  | active forever. |
| --- | --- | --- | --- | --- |

* Note: the # is replaced with a group number, e.g. Event.E0.Actions.A0. * Note:

current duration=0 means no action

2.8.10 Event Voice Alert Actions

Description: This group defines an action that runs a voice alert. [Event.E#.Actions.A#] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Protocol | VOICE | VOICE | 6/6 | Protocol. |
| Duration | 0 | An unsigned integer | 6/6 | How long the output should be active in seconds, 0 means that the output should be active for as long as the event is triggered. -1 means that the output should be active forever. |

* Note: the # is replaced with a group number, e.g. Event.E0.Actions.A0. * Note:

current duration=0 means no action

2.8.11 Event ICR Actions

| Description: This group defines an action that switch ICR to night mode. [Event.E#.Actions.A#] * Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Protocol | ICR | ICR | 6/6 | Protocol. |
| Duration | 0 | An unsigned integer | 6/6 | How long the output should be active in seconds, 0 means that the output should be active for as long as the event is triggered. |

|  |  |  |  | -1 means that the output should be active forever. |
| --- | --- | --- | --- | --- |

* Note: the # is replaced with a group number, e.g. Event.E0.Actions.A0. * Note:

current duration=0 means no action

2.8.12 Event ImgCtrl? Actions

Description: This group defines an action that switch image to b/w mode. [Event.E#.Actions.A#] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Protocol | ImgCtrl? | ImgCtrl? | 6/6 | Protocol. |
| Duration | 0 | An unsigned integer | 6/6 | How long the output should be active in seconds, 0 means that the output should be active for as long as the event is triggered. -1 means that the output should be active forever. |

* Note: the # is replaced with a group number, e.g. Event.E0.Actions.A0. * Note:

current duration=0 means no action

2.8.13 Event SAMBA Actions

| Description: This group defines an action that uploads files to an SAMBA server. [Event.E#.Actions.A#] * Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Protocol | SAMBA | SAMBA | 6/6 | Protocol. |
| Server |  | blank,S0 ... Sn (n = number of SAMBA event | 6/6 | SAMBA server ID. Refers to a parameter group under EventServers.SAMBA. |

|  |  | servers - 1) |  | Example: "F0" refers to the parameter group EventServers.SAMBA.S0. Server2 blank, S0 ... Sn (n = number of SAMBA event servers - 1) 6/6 Secondary SAMBA server ID. |
| --- | --- | --- | --- | --- |

* Note: the # is replaced with a group number, e.g. Event.E0.Actions.A0.

2.8.14 Event SD Actions

Description: This group defines an action that uploads files to an SD card. [Event.E#.Actions.A#] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Protocol | RECORD | RECORD | 6/6 | Protocol. |
| DiskID | SD_DISK | SD_DISK | 6/6 | SD ID. In this case, always be SD_DISK. |

* Note: the # is replaced with a group number, e.g. Event.E0.Actions.A0.

2.8.15 Recording Actions

Description: This group defines an action that record avi file to storage. [Event.E#.Actions.A#] *

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| type | R | R | 6/6 | Type. |
| Protocol | RECORD | RECORD | 6/6 | Protocol. |
| DiskID | SD_DISK | Storage ID | 6/6 | Storage ID. |

2.9 Event servers

2.9.1 EventServers.FTP.F#

Description: This group defines an FTP server that can be used by an event to upload files to.

[EventServers.FTP.F#]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Name | FTPServer# | A string | 6/6 | A descriptive name. |
| Address |  | An IP address or a host name | 6/6 | IP address or host name of the server |
| Login |  | A string | 6/6 | FTP user name |
| Password |  | A string | 6/6 | FTP password. |
| UploadPath |  | A string | 6/6 | Directory where uploaded files go. |
| Port | 21 | 0 ... 65535 | 6/6 | FTP port. |
| Passive | no | yes, no | 6/6 | Use passive FTP. |

2.9.2 EventServers.HTTP.H#

Description: This group defines an HTTP server that can be used by an event to upload files to or send notification messages to.

[EventServers.HTTP.H#]

| Parameter name | Default value | Valid values | Securit y level (get/se t) | Description |
| --- | --- | --- | --- | --- |
| Name | HTTPServe r# | A string | 4/4 | A descriptive name. |
| Address |  | A string | 4/4 | URL to the server, including name of CGI script to handle the request. Example: "http://192.168.254.10/cgi-bin/uploa d.cgi". |
| Login |  | A string | 4/4 | HTTP user name |

| Password |  | A string | 4/4 | HTTP password. |
| --- | --- | --- | --- | --- |
| Proxy |  | An IP addres s | 4/4 | Proxy address. |
| ProxyPort |  | 0 ... 65535 | 4/4 | Proxy port number. |
| ProxyLogin |  | A string | 4/4 | Proxy user name. |
| ProxyPasswo rd |  | A string | 4/4 | Proxy password. |

2.9.3 EventServers.TCP.T#

Description: This group defines a server that can be used by an event to send

TCP/IP notification messages to.

[EventServers.TCP.T#]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Name | TCPServer# | A string | 4/4 | A descriptive name. |
| Address |  | An IP address or a host name | 4/4 | IP address or host name of the server. |
| Port |  | 0 ... 65535 | 4/4 | TCP port number to use. |

2.10 Notification

2.10.1 Notify

Description: Parameters for sending notification when the IP address is changed.

2.10.2 Notify.FTP

Description: Parameters for notification by FTP. [Notify.FTP]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Enabled | no | yes,no | 6/6 | Enable sending of notifications by FTP. |
| HostName |  | A hostname | 6/6 | The IP address or hostname of the FTP server. |
| PassiveMode | no | yes,no | 6/6 | Establish a passive FTP connection. |
| Password |  | A string | 6/6 | The password for logging on to the FTP server. |
| Port | 21 | 0 ... 65535 | 6/6 | FTP port. |
| UploadPath |  | A string | 6/6 | The path to where the notification file will be saved on the FTP server. |
| UserName |  | A string | 6/6 | The username for logging on to the FTP server. |

2.10.3 Notify.HTTP

| Description: Parameters for notification by HTTP. [Notify.HTTP] Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Custom |  | A string (URL-encoded string with a leading & sign) | 6/6 | Additional parameters to Id.HTTPUrl. |
| Enabled | no | yes,no | 6/6 | Enable sending of HTTP GET request to a URL. |
| Password |  | A string | 6/6 | The password to Id.HTTPUrl . |
| Proxy |  | A string | 6/6 | The proxy server to pass when connecting to Id.HTTPUrl. |

| ProxyPass |  | A string | 6/6 | The password for Id.HTTPProxy. |
| --- | --- | --- | --- | --- |
| ProxyPort |  | A string | 6/6 | The port the Id.HTTPProxy listens to. |
| ProxyUser |  | A string | 6/6 | The username for Id.HTTPProxy. |
| Url | http:// | A URL | 6/6 | The URL to the cgi-script that will handle the information. |
| UserName |  | A string | 6/6 | The username for the Id.HTTPUrl. |

2.10.4 Notify.SMTP

Description: Parameters for notifications by e-mail. [Notify.SMTP]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Enabled | no | yes,no | 6/6 | Enable sending notifications by e-mail. |
| Subject |  | A string | 6/6 | The message to include in the "Subject" field in the e-mail notification. |
| ToEmail |  | An e-mail address or a list of e-mail addresses separated by ; | 6/6 | The recipient(s) for the e-mail. |

2.11 Properties

about product dependent functionality and functionality that have no ordinary parameters. All user levels should be able to access the property parameters.

Note: The Properties parameters are product dependent. If a parameter does not exist, the functionality is not supported.

2.11.1 Properties.API

[Properties.API.HTTP]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Version |  | An unsigned integer | 0/6 | The supported HTTP API version (only the first digit). |

2.11.2 Properties.Audio

[Properties.Audio]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Audio |  | yes,no | 0/6 | The product has audio support. |
| Format |  | A string | 0/6 | The supported formats separated by commas, e.g. g711,g721,g723. |

2.11.3 Properties.Firmware

[Properties.Firmware]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| BuildDate |  | A string | 0/6 | The build date for the current firmware in use. |
| BuildNumber |  | An unsigned integer | 0/6 | The build number for the current firmware in use. |
| Version |  | A string | 0/6 | The firmware version in use. |

2.11.4 Properties.GuardTour

[Properties.GuardTour]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| GuardTour |  | yes,no | 0/6 | The product has Guard Tour support. |

2.11.5 Properties.HTTPS

[Properties.HTTPS]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| HTTPS |  | yes, no | 0/6 | The product has HTTPS support. |

2.11.6 Properties.Image

[Properties.Image]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Format |  | A string | 0/6 | The supported formats separated by commas. E.g. jpg,mjpeg,mpeg4,bitmap. |
| Resolution |  | A string | 0/6 | The supported resolutions separated by commas. E.g. 4CIF,CIF,QCIF. Note: In products with widescreen support, only the currently available resolutions will be shown, i.e. enabling/disabling widescreen mode will affect the value of this parameter. |
| Rotation |  | A string | 0/6 | The supported image rotations separated by commas. E.g. 0,90,180,270. For products not supporting image rotation the value is 0. |
|  |  |  |  | 0 : Normal 90 : Mirror 180 : Mirror + Flip |

|  |  |  |  | 270 : Flip |
| --- | --- | --- | --- | --- |

2.11.7 Properties.ImageSource

[Properties.ImageSource]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| DCIris | no | yes, no | 0/6 | The product has DC Iris support. |
| Sensor | ov7670 | ov7670, ov9655, ov7725, tw9910, MT9M131, MT9M112, MT9V136 | 0/6 | Image sensor type of the product. |
| SensorType | CCD | CCD, CMOS | 0/6 | Image sensor type. |
| Anti-Blooming | yes | yes, no | 0/6 | The product has Anti-Blooming support. |
| WhiteBalance | yes | yes, no | 0/6 | The product has WhiteBalance support. |
| Exposure | yes | yes, no | 0/6 | The product has Exposure support. |
| PowerFrequence | yes | yes, no | 0/6 | The product has PowerFrequence support. |
| Outdoor | no | yes, no | 0/6 | The product has Outdoor support. |
| NightVision | no | yes, no | 0/6 | The product has night vision support. |
| MegaPixel | no | yes, no | 0/6 | The product has mega pixel support. |
| ColorLevel | yes | yes, no | 0/6 | The product has ColorLevel control. |

| Brightness | yes | yes, no | 0/6 | The product has Brightness control. |
| --- | --- | --- | --- | --- |
| Contrast | yes | yes, no | 0/6 | The product has Contrast control. |
| Sharpness | yes | yes, no | 0/6 | The product has Sharpness control. |
| ICR | yes | yes, no | 0/6 | The product has ICR control. |
| D-Brightness | yes | yes, no | 0/6 | The product has D-Brightness control. |
| BLC | yes | yes, no | 0/6 | The product has BLC control. |
| De-Noise | yes | yes, no | 0/6 | The product has De-Noise control. |

2.11.8 Properties.Motion

[Properties.Motion]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Motion |  | yes, no | 0/6 | The product has Motion Detection support. |

2.11.9 Properties.PTZ

[Properties.PTZ]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| PTZ |  | yes, no | 0/6 | The product has Pan Tilt support. |
| Zoom |  | yes, no | 0/6 | The product has Zoom support. |
| Focus |  | yes, no | 0/6 | The product has focus support. |
| AutoFocus |  | yes, no | 0/6 | The product has autofocus |

|  |  |  |  | support. |
| --- | --- | --- | --- | --- |
| DigitalPTZ |  | yes,no | 0/6 | The product has digital Pan Tilt Zoom support. |
| RS485 |  | yes,no | 0/6 | The product has RS485 support. |

2.11.10 Properties.Recorder

[Properties.Recorder]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Recorder |  | yes,no | 0/6 | The product has recorder support. |

2.11.11 Properties.System

[Properties.System]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| HardwareID |  | A string | 0/6 | The hardware ID for the product. |
| Language |  | A string | 0/6 | The current language in use. |
| SerialNumber |  | A string | 0/6 | The unit's serial number. |

2.11.12 Properties.Tampering

[Properties.Tampering]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Tampering | no | yes,no | 0/6 | The product has camera tampering support. |

2.11.13 Properties.TemperatureSensor

[Properties.TemperatureSensor]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| TemperatureSensor |  | yes,no | 0/6 | The product has a temperature sensor. |

2.11.14 Properties.Wireless

[Properties.Wireless]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Wireless |  | yes,no | 0/6 | The product has wireless support. |

2.11.15 Properties.IR

[Properties.IR]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| IR |  | yes,no | 0/6 | The product has IR support. |

2.11.16 Properties.VideoOut

[Properties.VideoOut]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| VideoOut |  | yes,no | 0/6 | The product has video output support. |

2.11.17 Properties.HardwareReset

[Properties.HardwareReset]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| HardwareReset |  | yes,no | 0/6 | The product has video |

|  |  |  |  | HardwareReset support. |
| --- | --- | --- | --- | --- |

2.11.18 Properties.LED

[Properties.LED]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Usage |  | yes,no | 0/6 | The product has led control support. |

2.11.19 Properties.DynamicParam

| Description: Show the maximum size of dynamic parameters. [Properties.DynamicParam] Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| MaxTour | 16 | 1 ~ 32 | 0/6 | Maximum size of Tour. |
| MaxPresetOfTour | 32 | 1 ~ 64 | 0/6 | Maximum size of preset of each tour. |
| MaxPreset | 128 | 1 ~ 256 | 0/6 | Maximum size of preset. |
| MaxEvent | 30 | 1 ~ 64 | 0/6 | Maximum size of event. |
| MaxEventAction | 13 | 1 ~ 16 | 0/6 | Maximum size of event action. |
| MaxEventServerFTP | 16 | 1 ~ 32 | 0/6 | Maximum size of FTP server. |
| MaxEventServerTCP | 16 | 1 ~ 32 | 0/6 | Maximum size of TCP server. |
| MaxEventServerHTTP | 16 | 1 ~ 32 | 0/6 | Maximum size of HTTP server. |
| MaxEventServerSAMBA | 16 | 1 ~ 32 | 0/6 | Maximum size of SAMBA server. |

| MaxMotion | 10 | 1 ~ 10 | 0/6 | Maximum size of motion window. |
| --- | --- | --- | --- | --- |

2.12 Web interface

2.12.1 Layout

Description: Parameters that define how the live view page looks and works. [Layout]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| DefaultVideoFormat | mjpeg | mjpeg,mpeg4 | 0/6 | The default video format to be shown. |

2.13 Wireless

2.13.1 Network.Wireless

Description: Parameters for wlan (802.11b) settings. To setup these parameters, check your wireless access point and use same settings in your device. These parameters are also in a text file, CONFIG.TXT, accessible over USB.

[Network.Wireless]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| ESSID |  | A string, up to 32 alphanumeric characters are accepted, but no space characters. | 6/6 | The name of the wireless network the device is configured for. The name must be exactly the same as used in the wireless access point, or the connection will not be established. |

|  |  |  |  | ESSID is called SSID by some vendors. Note: Leaving this parameter blank means that the device will attempt to access the nearest open network. |
| --- | --- | --- | --- | --- |
| Mode | managed | managed,ad-hoc,auto | 6/6 | The type of wireless network to associate with, managed (using an access point), ad-hoc (not using an access point) or auto. Auto should only be used in factory default situations. |
| SecurityMode | none | none, wpa-psk, wep | 6/6 | Security mode. |
| Channel | auto | auto, 1~14 | 6/6 | channel |

2.13.2 Network.Wireless.W0

Description: Parameters for wlan (802.11b) settings. To setup these parameters, check your wireless access point and use same settings in your device. These parameters are also in a text file, CONFIG.TXT, accessible over USB.

[Network.Wireless.W0]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Method | WPA-PSK | WPA-PSK | 6/6 | Security method. |
| Enabled | no | yes,no | 6/6 | Enable/disable WPA-PSK security. |
| GenerationMethod | psk-phrase | psk-phrase,psk-hex | 6/6 | Defines which of the parameters |

|  |  |  |  | below that is used as pre-shared key. |
| --- | --- | --- | --- | --- |
| Passphrase |  | A string, 8 - 63 ascii chars | 6/6 | The passphrase must match the passphrase in the wireless access point. When passphrase is used, the parameter GenerationMethod must be set to "psk-phrase". |
| Key |  | 64 Hex chars | 6/6 | The key must match the key in the wireless access point. When key is used, the parameter GenerationMethod must be set to "psk-hex". |

2.13.3 Network.Wireless.WEP

Description: Parameters for WEP encryption of wlan (802.11b). To setup these parameters, check your wireless access point and use same settings in your device. These parameters are also in a text file, CONFIG.TXT, accessible over USB.

[Network.Wireless.WEP]

| Parameter name | Defaul t value | Valid values | Security level (get/set ) | Description |
| --- | --- | --- | --- | --- |
| Authentication | open | open,shared | 6/6 | The WEP authentication method. |
| Enabled | no | yes,no | 6/6 | Enable/disable WEP |

|  |  |  |  | encryption over the wireless link. |
| --- | --- | --- | --- | --- |
| KeyLength | 128 | 64,128 | 6/6 | Set the strength of the wireless encryption. |
| GenerationMetho d | manua l | manual,ASCII,passphra se | 6/6 | This parameter must tell which of the parameters below that is used and how to interpret them. Manual hex always works, while ASCII and passphrase are easier to remember. |
| Passphrase |  | A string of 31 characters | 6/6 | The passphrase must match the passphrase in the wireless access point. When passphrase is used, parameter GenerationMethod must be set to "passphrase" |
| Key1 |  | A string | 6/6 | The keys must match the keys in the wireless access point. They could either be in hex format or in ASCII and must match parameter GenerationMethod, see above. Hex (manual) - the string must be |

|  |  |  |  | exactly 10 hex characters for 64-bit WEP and 26 hex characters for 128-bit WEP. (Hex chars are 0123456789ABCDE F) ASCII - The string must be exactly 5 characters for 64-bit WEP and 13 characters for 128-bit WEP. |
| --- | --- | --- | --- | --- |
| Key2 |  | A string | 6/6 |  |
| Key3 |  | A string | 6/6 |  |
| Key4 |  | A string | 6/6 |  |
| ActiveKey | 1 | 1 ... 4 | 6/6 | Which key to use when transmitting. The key should be changed every now and then, to increase the security of the wireless connection. |

2.14 Audio

2.14.1 Audio

| Description: Common audio parameters used for all audio configurations. [Audio] Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| DuplexMode | get | get,full | 6/6 | How the audio should be |

|  |  |  |  | transferred. full = Full duplex - simultaneous two-way audio. Transmit and receive audio at the same time. half = Half duplex - non simultaneous two-way audio. Audio only allowed in one direction at a time. post = Simplex. Audio to the server. get = Simplex. Audio from the server. |
| --- | --- | --- | --- | --- |
| NbrOfConfigs | 1 | 1 | 6/6 | Number of audio configurations. |

2.14.2 Audio.A#

Description: Audio configuration. The name of the audio configuration, which audio source it is connected to and whether it should be enabled or not.

[Audio.A0]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Enabled | yes | yes,no | 6/6 | Enable/disable this audio configuration. |
| HTTPMessageType | singlepart | singlepart,multipart | 1/6 | How the audio should be streamed. Some proxies require multipart streaming. |
| Name |  | A string | 6/6 | The name of the configuration. |
| Source | 0 | An integer | 6/6 | Which AudioSource.A# this Audio.A# configuration is |

|  |  |  |  | connected to. |
| --- | --- | --- | --- | --- |

2.14.3 AudioSource

Description: The number of audio sources (audio inputs/chips). [AudioSource]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| NbrOfSources | 1 | An unsigned integer | 0/6 | Number of audio sources (audio input/chips). |

2.14.4 AudioSource.A#

| Description: Parameters for each audio source (audio input/chip). [AudioSource.A0] Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| AudioEncoding | g721 | g711,g721,g723 | 6/6 | The audio codec to use. |
| BitRate | 32000 | g711: 64000 g721: 24000 g726: 32000 | 6/6 | The output bit rate (bits per second) from the encoder. |
| SampleRate | 8000 | 8000 16000 | 6/6 | The clock rate for the audio sampling. |
| InputGain | 0 | mute, -60, -57, -54 ... -3, 0, 3, 6 ... 21, 24, 27, 30 | 6/6 | Gain settings in dB for sound received from client. |
| OutputGain | 0 | mute, -60, -57, -54 ... -3, 0, 3, 6 ... 21, 24, 27, 30 | 6/6 | Gain setting in dB for sound transmitted to client(s). |
| MuteOnPT | no | yes, no | 6/6 | Reduce audio input gain while pt |

|  |  |  |  | control. |
| --- | --- | --- | --- | --- |

* Note: the # is replaced with a group number starting from zero, e.g. AudioSource.A0

1 Product-dependent. Check the corresponding Property parameter.

2 Product-dependent.

2.15 Recorder

2.15.1 Recorder

Description: Common recorder parameters used for all recorder configurations. [Recorder]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Type | avi-jpeg | avi-h264,avi-jpeg,jpeg | 6/6 | The record file format. |

2.16 Storage

| Description: Common storage parameters used for all storage configurations. [Storage.S0] Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| MountDir | /mnt/sd | /mnt/sd | 6/6 | readonly |
| DiskID | SD_DISK | SD_DISK | 6/6 | readonly |
| DeviceNode | /tmp/cpesda1 | /tmp/cpesda1 | 6/6 | readonly |
| CleanupLevel | 95 | 1 ~ 100 | 6/6 | Remove oldest recordings when disk is CleanupLevel% full |
| CleanupMaxAge | 1 | 1 ~ 999 | 6/6 | Remove recordings older than CleanupMaxAge |

|  |  |  |  | days |
| --- | --- | --- | --- | --- |
| CleanupPolicyActive | none | none, fifo | 6/6 | Enable automatic disk cleanup |
| FileSystem | vfat | vfat | 6/6 | sd card filesystem |
| Locked | no | yes, no | 6/6 | Lock disk |
| MountOnBoot | yes | yes, no | 6/6 | Auto mount on boot |

2.17 StreamProfile

Description: streaming profile description

[StreamProfile.S#]

| Parameter name | Default value | Valid values | Security level (get/set) | Description |
| --- | --- | --- | --- | --- |
| Name |  | string | 6/6 |  |
| Description |  | string | 6/6 |  |
| Parameters |  | string | 6/6 | * |
| Default.Name |  | string | 6/6 |  |
| Default.Description |  | string | 6/6 |  |
| Default.Parameters |  | string | 6/6 |  |

* Parameters field fas following parameters:

Format:

videocodec=mpeg4&resolution=vga&compression=100&fps=30&vide okeyframeinterval=30&videobitrate=1024

1. videocodec: h264,  mpeg4,  mjpeg

2. resolution:  sxga, vga, qvga,   qqvga,   4cif, cif, qcif

(hardware dependent)

3. compression: 1  ~ 100

4. fps: 1  ~ 30  (hardware dependent)

5. videokeyframeinterval: 1  ~ 60

6. videobitrate: 32  ~ 4096  (hardware dependent)